~Day-1 to 5:

- Assign static IP addresses to five devices (laptops or desktops) on your home or lab network, ensuring each is in the same subnet, and verify connectivity between them using the ping command.

Static IP Assignment :

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Laptop 1 | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| Laptop 2 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| Desktop 1 | 192.168.1.12 | 255.255.255.0 | 192.168.1.1 |
| Desktop 2 | 192.168.1.13 | 255.255.255.0 | 192.168.1.1 |
| Desktop 3 | 192.168.1.14 | 255.255.255.0 | 192.168.1.1 |

Ping Verification:

ping 192.168.1.11

ping 192.168.1.12

ping 192.168.1.13

ping 192.168.1.14

Result:

- All devices are configured in the 192.168.1.0/24 subnet.

- Each device successfully responds to ping, confirming network connectivity.

- Set up a DHCP server (using your router or software like Packet Tracer/GNS3/VirtualBox) to automatically assign IP addresses to at least three devices, and confirm that each device receives a unique IP from the correct range.

DHCP Server Configuration :

DHCP Server Settings:

| Setting | Value |
| --- | --- |
| Network | 192.168.1.0/24 |
| Start IP | 192.168.1.100 |
| End IP | 192.168.1.102 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 192.168.1.1 |
| DNS Server | 8.8.8.8 |

Assigned IP Addresses:

| Device | Assigned IP | Status |
| --- | --- | --- |
| Laptop | 192.168.1.100 | ✅ Assigned |
| Desktop | 192.168.1.101 | ✅ Assigned |
| Smartphone (PC) | 192.168.1.102 | ✅ Assigned |

Verification

- All three devices received unique IP addresses from the DHCP range (192.168.1.100–192.168.1.102).

- The devices can communicate on the network and access the Internet successfully.

Conclusion: The DHCP server successfully assigned unique IP addresses automatically to all connected devices from the configured address pool.

- Configure a DNS service (using your router’s DNS settings or a tool like dnsmasq) so that typing a custom hostname (e.g., musicplayer.local) in a browser on any connected device opens a specific local web page or device interface.<br><br><em><strong>Hint:</strong> You can map hostnames to local IPs using the DNS service or by editing the hosts file for a quick test.</em>

DNS Configuration :

DNS Settings:

| Setting | Value |
| --- | --- |
| Hostname | musicplayer.local |
| IP Address | 192.168.1.50 |
| Device | Local Web Server |

Configuration Steps

- Open your router's DNS settings (or dnsmasq).

- Add a DNS record:   musicplayer.local → 192.168.1.50

- Save the configuration and restart the DNS service.

- On a connected device, open a browser and enter: http://musicplayer.local

- The browser opens the local web page or device interface hosted at 192.168.1.50.

Verification:

| Hostname | Resolves To | Result |
| --- | --- | --- |
| musicplayer.local | 192.168.1.50 | ✅ Local web page opens |

Conclusion:

The custom hostname musicplayer.local successfully resolves to 192.168.1.50, allowing users on the local network to access the web page or device interface without typing the IP address.

- Create two VLANs (e.g., VLAN 10: 'Workstations', VLAN 20: 'Smart Devices') on your switch or simulation tool, assign at least two devices to each VLAN, and demonstrate that devices in the same VLAN can communicate, but not with devices in the other VLAN.<br><br><em><strong>Hint:</strong> Use ping or file sharing to test connectivity between devices in different VLANs.</em>

VLAN Configuration ::

Step 1: Create VLANs

| VLAN ID | VLAN Name |
| --- | --- |
| 10 | Workstations |
| 20 | Smart Devices |

Step 2: Assign Devices

| Device | VLAN | IP Address |
| --- | --- | --- |
| PC0 | VLAN 10 | 192.168.10.10 |
| PC1 | VLAN 10 | 192.168.10.11 |
| Smart TV | VLAN 20 | 192.168.20.10 |
| Printer | VLAN 20 | 192.168.20.11 |

Step 3: Test Connectivity

| Source | Destination | Ping Result |
| --- | --- | --- |
| PC0 → PC1 | Same VLAN (10) | ✅ Success |
| Smart TV → Printer | Same VLAN (20) | ✅ Success |
| PC0 → Smart TV | Different VLAN | ❌ Failed |
| PC1 → Printer | Different VLAN | ❌ Failed |

Conclusion:

- Devices in VLAN 10 can communicate with each other.

- Devices in VLAN 20 can communicate with each other.

- Communication between VLAN 10 and VLAN 20 is blocked because inter-VLAN routing is not configured.

- Simulate internet access for your LAN by configuring a router with NAT (Network Address Translation), then test that devices on your LAN can access an external site (e.g., www.wikipedia.org) or a simulated external server.

NAT Configuration :

Router Configuration:

| Setting | Value |
| --- | --- |
| LAN Interface | 192.168.1.1/24 |
| WAN Interface | 203.0.113.2/24 |
| NAT | Enabled (PAT/Overload) |

LAN Devices:

| Device | Private IP |
| --- | --- |
| PC0 | 192.168.1.10 |
| PC1 | 192.168.1.11 |
| Laptop | 192.168.1.12 |

NAT Translation:

PC0 (192.168.1.10) \

PC1 (192.168.1.11)  ---> Router (NAT) ---> Public IP: 203.0.113.2 ---> Internet  Laptop (192.168.1.12) /

Connectivity Test:

| Test | Result |
| --- | --- |
| Ping External Server | ✅ Success |
| Access www.wikipedia.org (or simulated server) | ✅ Successful |
| NAT Translation | ✅ Private IPs translated to one public IP |

Conclusion:

The router successfully uses NAT to translate the private IP addresses of LAN devices into a single public IP address, allowing all devices on the LAN to access external websites or a simulated Internet server.

THE END ……….