~Day-5:

- Simulate a DNS resolution failure on your local machine by editing the hosts file to point www.spotify.com to a wrong IP address, then try to ping the domain and document the error message you receive.

Safe Simulation:

Step 1: Edit the hosts file and add:

192.168.1.250    musicmania.in

Step 2: Save the file and run:

ping musicmania.in

Example Output:

Pinging musicmania.in [192.168.1.250] with 32 bytes of data:

Request timed out.

Request timed out.

Request timed out.

Request timed out.

Ping statistics for 192.168.1.250:

Packets: Sent = 4, Received = 0, Lost = 4 (100% loss)

Explanation:

- The hosts file overrides normal DNS resolution.

- Because the domain was mapped to an incorrect IP address (192.168.1.250), the ping failed with "Request timed out."

- Configure a DHCP server in a network simulator (such as Cisco Packet Tracer or GNS3) and intentionally set an incorrect default gateway; connect a client device and verify the assigned gateway, then describe how you would identify and fix this issue.

DHCP Misconfigured Default Gateway (Cisco Packet Tracer):

Step 1: Configure the DHCP Server (Incorrect Gateway)

| Setting | Value |
| --- | --- |
| Server IP | 192.168.1.2 |
| DHCP Pool | LAN |
| Start IP | 192.168.1.100 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway (Incorrect) | 192.168.1.254 |
| DNS Server | 8.8.8.8 |

Step 2: Connect the Client

- Connect PC0 to the switch.

- On PC0, go to Desktop → IP Configuration.

- Select DHCP.

Step 3: Verify the Assigned Configuration

| Parameter | Assigned Value |
| --- | --- |
| IP Address | 192.168.1.100 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 192.168.1.254 (Incorrect) |

Problem Observed:

- The PC receives an IP address successfully.

- The default gateway is incorrect.

- The PC cannot reach devices outside the local network.

How to Identify the Issue:

- Check the PC's IP Configuration and verify the assigned gateway.

- Try pinging the default gateway. The ping will fail.

- Open the DHCP server settings and compare the configured gateway with the router's actual IP address.

Fix the Issue:

- Change the DHCP Default Gateway from:  192.168.1.254

to the router's correct IP address, for example:   192.168.1.1

- Renew the client's DHCP lease (or click DHCP again in Packet Tracer).

Verification After Fix:

| Parameter | Correct Value |
| --- | --- |
| IP Address | 192.168.1.100 |
| Default Gateway | 192.168.1.1 ✅ |

Conclusion:

The problem was caused by an incorrect default gateway configured on the DHCP server. After changing it to the correct router IP address and renewing the DHCP lease, the client was able to communicate with external networks successfully.

- Given a scenario where devices on your simulated network are unable to access websites by name but can by IP address, use the nslookup or dig tool to diagnose the DNS problem and write down the troubleshooting steps and solution.

Scenario:

The devices on the network can access websites using IP addresses (e.g., 142.250.183.78) but cannot access them by domain name (e.g., www.google.com). This indicates a DNS resolution problem.

Step 1: Test with nslookup

nslookup www.google.com

Example Output:

Server:  UnKnown

Address: 192.168.1.5

*** Can't find www.google.com: Server failed

Step 2: Troubleshooting Steps

- Run ipconfig /all and verify the DNS Server address.

- Run nslookup www.google.com to check whether DNS resolution is working.

- Ping the DNS server (e.g., ping 192.168.1.5) to ensure it is reachable.

- Check the DNS server configuration and verify that the DNS service is running.

- Flush the DNS cache using:  ipconfig /flushdns

- Test again with:   nslookup www.google.com

Solution:

- Correct the DNS server IP address if it is incorrect.

- Restart the DNS service if it is stopped.

- Flush the DNS cache and test again.

Result:

After fixing the DNS server configuration, nslookup successfully resolves the website name, and users can access websites using domain names instead of only IP addresses.

- A device in your simulated network is not receiving an IP address from the DHCP server and shows an APIPA address (169.x.x.x). List the possible causes for this and document the step-by-step process you would use to resolve it.<br><br><em><strong>Hint:</strong> Check for network connectivity, DHCP scope exhaustion, and server availability.</em>

Scenario:

A PC in the simulated network displays an APIPA address (169.254.x.x) instead of a valid DHCP IP address. This means it could not obtain an IP address from the DHCP server.

Possible Causes:

- DHCP server is offline or the DHCP service is stopped.

- Network cable or switch connection is faulty.

- DHCP IP pool (scope) is exhausted.

- Incorrect VLAN or network configuration.

- Firewall or security settings are blocking DHCP traffic.

Troubleshooting Steps:

- Check that the PC is properly connected to the network.

- Verify that the DHCP server is powered on and the DHCP service is running.

- Check the DHCP pool to ensure there are available IP addresses.

- Renew the IP address using:

- ipconfig /release ipconfig /renew

- Verify the assigned IP address:  ipconfig

- Test connectivity to the DHCP server using:  ping <DHCP_Server_IP>

Solution:

- Start or restart the DHCP server if it is unavailable.

- Increase the DHCP scope if all addresses are allocated.

- Fix any cable, switch, or VLAN configuration issues.

- Renew the client's IP address after correcting the problem.

Result:

After resolving the issue, the PC receives a valid IP address (e.g., 192.168.1.100) from the DHCP server instead of an APIPA (169.254.x.x) address, and network connectivity is restored.

- Create a table documenting at least three common DNS issues and three common DHCP issues you encountered or researched, including symptoms, possible causes, and recommended solutions for each.

| Type | Issue | Symptoms | Solution |
| --- | --- | --- | --- |
| DNS | DNS server down | Website name not opening | Restart DNS service |
| DNS | Wrong DNS record | Wrong IP returned | Correct the DNS record |
| DNS | DNS cache issue | Old IP is used | Run ipconfig /flushdns |
| DHCP | APIPA (169.254.x.x) | No valid IP address. | Restart DHCP and renew IP |
| DHCP | IP pool exhausted | New device gets no IP. | Increase DHCP scope |
| DHCP | Wrong gateway | No Internet access | Configure the correct gateway |

THE END……..